# Smart Light Proof of Concept

Ambient light sensing with automatic phone flashlight control

Purpose. This proof of concept demonstrates a complete sensor-to-action loop: an Android phone reports ambient illumination, a Python service evaluates the reading against a configurable lux threshold, and Nikki Streams receives a flashlight command. A browser dashboard shows the live reading and requested flashlight state.

# Implementation at a glance

| Layer | Implementation |
| --- | --- |
| Phone and sensor | Android phone running Nikki Streams; Light sensor enabled; flashlight service exposed through the session. |
| Device connection | nikki-streams Python SDK using the current Streams socket address and a session ID. |
| Application bridge | Python 3.12, aiohttp HTTP/WebSocket server, bound to 0.0.0.0:8080. |
| Dashboard | index.html, responsive phone mock-up, threshold control, automatic/manual controls, and live status. |
| Current endpoint | Streams WebSocket ws://10.172.21.252:3000; dashboard http://10.172.21.60:8080/. |

# Data and command path

PHONE LIGHT SENSOR → Streams session → Python callback lux reading threshold check ↓ PHONE FLASHLIGHT ← Streams SDK command ← local state ↓ BROWSER DASHBOARD ← aiohttp WebSocket /ws

Network note. The laptop Wi-Fi address was 10.172.21.60 (DHCP). The address 192.168.56.1 belongs to the VirtualBox host-only adapter and is not the dashboard address. Laptop IP addresses can change after reconnecting.

# Technical Design

The bridge separates device communication from the browser interface. The browser uses a local WebSocket at /ws to send threshold and mode changes. The Python service uses the Nikki SDK to communicate with the phone’s Streams server; browser code does not implement the Streams protocol directly.

# Sensor callback and threshold decision

The Streams callback contains event data. The bridge reads the Light value in lux and compares it with the active threshold. Automatic mode requests the flashlight on below the threshold and off at or above it.

# server.py — simplified excerpt if event.type == "data": payload = event.data or {} sensor_data = payload.get("data", payload) lux = sensor_data.get("Light", sensor_data.get("light")) if isinstance(lux, (int, float)): state["lux"] = float(lux) if state["automatic"]: loop.create_task( set_flashlight(state["lux"] < state["threshold"]) )

# Streams SDK connection and actuation

The SDK is initialized once with the session ID. A device is then created using the phone socket host and port. The flashlight helper sends commands only after the device reports a connected state.

# Connection setup (server.py) SDK.initialize(SESSION_ID) device = SDK.create_device(PHONE_ADDRESS, phone_callback) await device.connect() # Actuation await device.set_light_status(True) # turn on await device.set_light_status(False) # turn off

# Dashboard messaging

The page opens a WebSocket to the local bridge using the same host and port as the loaded page. This lets the page work from either the laptop or another device on the same network.

// index.html — local dashboard channel const protocol = location.protocol === "https:" ? "wss" : "ws"; const socket = new WebSocket(`${protocol}://${location.host}/ws`); socket.send(JSON.stringify({ type: "threshold", value: 20 }));

# Example sensor event

The callback unwraps the event payload and reads the Light field. A current reading and timestamp are then sent to the dashboard state.

{ "type": "data", "data": { "time": 1792041106742, "data": { "Light": 463.65 } } }

# Validation and Operation

# Observed proof-of-concept results

| Check | Observed result |
| --- | --- |
| Network reachability | Laptop received Wi-Fi IPv4 10.172.21.60 on the same 10.172.21.x subnet as the Streams host. |
| Device session | Dashboard reported Phone connected; bridge state connected=true with no error. |
| Live sensor | Ambient reading reported as 463.65 lux. |
| Bright-condition behavior | At a 20-lux threshold, the flashlight state was off. |
| Dark-condition simulation | Temporarily raising the threshold to 500 lux caused an on command at 463.65 lux. |
| Restore check | Restoring the threshold to 20 lux caused an off command; original threshold was restored. |

Interpretation. The live sensor-to-command path was verified. The SDK does not provide a separate hardware-state acknowledgement, so the dashboard’s flashlight indicator represents successful command delivery/state held by the bridge.

# Run instructions

Set-Location "C:\Users\Kiran Rao\Downloads\LIGHT" .\.venv\Scripts\Activate.ps1 pip install -r requirements.txt $env:PHONE_ADDRESS = "10.172.21.252:3000" python server.py

Open http://10.172.21.60:8080/ on the laptop or phone while both devices share the same network.

If the laptop IP changes, run ipconfig and use the IPv4 address under Wireless LAN adapter Wi-Fi.

In the dashboard, enter the current Streams session ID and socket address, then select Connect phone. The session ID is intentionally not stored in this document.

Enable the Light sensor in Streams. The default automation threshold is 20 lux; change it in the dashboard as needed.

# Dependencies and current limits

| Dependency | Purpose |
| --- | --- |
| Python 3.10+ | Runs the asynchronous bridge; this project was run with Python 3.12. |
| aiohttp | Serves the dashboard and the local /ws WebSocket. |
| nikki-streams | Connects to the phone session, receives sensor events, and sends flashlight commands. |

The Streams Light sensor must be enabled, and the phone socket address must be current and reachable from the laptop.

The bridge retries a failed or lost phone connection every three seconds. The session ID is provided at runtime through the dashboard.

The threshold logic has no hysteresis; readings fluctuating around the threshold can switch the flashlight repeatedly.